Semester Project Theme Description

Course: Artificial Intelligence and Autonomous Vehicles

Project Title: Advanced Path Tracking in Noisy Environments using Model Predictive Control and Extended Kalman Filtering

Team Members

| Name | Neptun Code | Assigned Theme |
| --- | --- | --- |
| <Student Name 1> | <Neptun Code 1> | Control: Model-Predictive Control (MPC) |
| <Student Name 2> | <Neptun Code 2> | State Estimation: Localization (EKF) |
| <Student Name 3> | <Neptun Code 3> | Simulation: 2D Kinematic Simulation |

1. Project Objective

The objective of this project is to build a modular, 2D simulation of an autonomous vehicle navigating a predefined trajectory under realistic conditions. Real-world autonomous systems rely on imperfect, noisy sensor data. This project demonstrates how advanced state estimation can filter noisy GPS/velocity data to find the vehicle's true state, and how optimal control theory can use that estimated state to smoothly drive the vehicle along a target path.

2. Selected Themes and Methodology

This project integrates three distinct themes from the course syllabus, implemented entirely in Python:

Theme A: Control - Model Predictive Control (MPC)

Implemented by: <Student Name 1> We will implement an MPC algorithm to calculate the optimal steering angle and acceleration for the vehicle. The MPC will utilize a receding horizon approach, predicting the vehicle's future states over a finite time window to minimize cross-track error while strictly adhering to physical actuation constraints (e.g., maximum steering angle and velocity).

Theme B: State Estimation - Localization via Extended Kalman Filter (EKF)

Implemented by: <Student Name 2> To simulate real-world conditions, the vehicle's raw sensor data will be intentionally corrupted with Gaussian noise. We will implement an Extended Kalman Filter to fuse noisy simulated GPS (X, Y position) and dead-reckoning (velocity, yaw rate) data. The EKF will linearize the non-linear kinematic bicycle model using Jacobian matrices to output a cleaned, highly accurate state estimate for the controller.

Theme C: Simulation - 2D Environment and Vehicle Kinematics

Implemented by: <Student Name 3> We will build a custom 2D simulation environment using matplotlib and numpy. This module will act as the "ground truth." It will simulate the physics of a kinematic bicycle model responding to the MPC's control inputs, generate the target reference trajectories (e.g., a figure-8 track or lane-change maneuver), inject mathematical noise into the sensor readings, and provide the visual animation of the system.

3. Expected Quantitative Results

To fulfill the requirements for the final passing grade and the 20-page documentation, the system will output the following quantitative metrics:

- Estimation Accuracy: A comparative plot and Root Mean Square Error (RMSE) calculation of the True Path vs. Noisy Sensor Path vs. EKF Estimated Path.

- Tracking Performance: Descriptive statistics of the Cross-Track Error (CTE) measured across different target velocities and track curvatures.

- Control Effort: Analysis of the steering angle variance over time to prove the smoothness of the MPC formulation compared to baseline parameters.

- Computational Efficiency: Average execution time per loop of the MPC solver to verify real-time feasibility.